# Communication between MES and PLC

# 1. General

## Naming Conventions

The following naming conventions are utilized throughout the integration documentation:

- Mill System (MES): The central manufacturing execution server.

- PLC: The programmable logic controller.

- Label Printer (LBL): The physical printing hardware for product identification.

## PLCs, PCs, Names, IP-addresses and Ports

| Name | IP-address | Port(s) | Notes |
| --- | --- | --- | --- |
| MES |  |  |  |
| PLC |  |  |  |
| Labeling printer | Installed as a Windows printer | — |  |

- Mask: 255.255.255.0

- Gateway: Place

# 2. Interface and Communication Details

- Network Type: Ethernet

- Protocol: TCP/IP Sockets

- Data Transfer Type: TCP Data Streams (ASCII-strings)

- Message Type: ASCII-string with separator character between data fields.

## General Specifications

- All data fields must be readable and printable ASCII-characters.

- Data fields are separated by a pipe ("|") character (ASCII 124 / HEX 7C).

- Length of the message (240 chars) must be the same in each message (MES ↔ PLC).

- Numeric (integer, float) values must be converted to ASCII-characters.

- Decimal numbers (digits after decimal point) must be separated by a dot (".").

- Character data fields are padded with trailing blanks.

- Numeric data fields are padded with leading zeros.

- Message starts with a Start of TeXt <STX> (HEX 02).

- Message ends with an End of TeXt <ETX> (HEX 03).

- Message Buffering: No message buffering, i.e. link down time data will not be sent later.

- Data Format: <STX>MessageID|Message|Data1|Data2|…|Data n|<ETX>

  - Message ID (4 chars) = Message ID of Message

  - Example: "WR01|0010|01|0000987777|…|"

## Connection Architecture

- Client (active): PLC

- Server (listen): MES

- Client side opens and maintains the connection, so there is no need to reopen the connection before each transmit.

- The client side closes and reopens the connection if it fails to transmit messages successfully.

# 3. Messages when Product at Conveyor Starting Position #1

## Operation & Sequence

- PLC sends a set data query to the MES.

- MES responds with the set data message.

Communication Flow:

- PLC -> MES: (1) Sends RW01 (Set Data Query)

- MES -> PLC: (2) Sends MW01 (Set Data at stopper)

## 3.1 Communication

### 3.2 "RW01" Set Data Query (MES ↔ PLC)

| No | Description | Length | Format | Comment |
| --- | --- | --- | --- | --- |
| — | Start of MSG Char | 1 | STX | HEX 02 |
| 1 | Message ID | 4 | RW01 |  |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| — | End of MSG Char | 1 | ETX | HEX 03 |

### 3.3 "MW01" Set Data at stopper (MES ↔ PLC)

| No | Description | Length | Format | Comment |
| --- | --- | --- | --- | --- |
| — | Start of MSG Char | 1 | STX | HEX 02 |
| 1 | Message ID | 4 | MW01 |  |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| 2 | Product ID (Unit ID) | 12 | 0123456789aa |  |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| — | End of MSG Char | 1 | ETX | HEX 03 |

# 4. Messages when Product at Scale Conveyor

## Operation

- PLC measures product weight.

- PLC sends the product id and measured weight to MES.

- MES checks that measures are within tolerances.

- MES sends an acknowledgement message.

Communication Flow:

- PLC -> MES: (1) "RG01" Roll at scale conveyor (Product ID & Measured Weight)

- MES -> PLC: (2) "MG01" Roll data acknowledgement

### "RG01" Roll is identified at scale conveyor (MES ↔ PLC)

| No | Description | Length | Format | Comment |
| --- | --- | --- | --- | --- |
| — | Start of MSG Char | 1 | STX | HEX 02 |
| 1 | Message ID | 4 | RG01 |  |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| 2 | Product ID (Unit ID) | 12 | 0123456789af |  |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| 3 | Product weight | 4 | 0000-9999 | [kg] |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| — | End of MSG Char | 1 | ETX | HEX 03 |

### "MG01" Roll Data acknowledgement (MES ↔ PLC)

| No | Description | Length | Format | Comment |
| --- | --- | --- | --- | --- |
| — | Start of MSG Char | 1 | STX | HEX 02 |
| 1 | Message ID | 4 | MG01 |  |
| — | Delimiter | 1 | ¦ | HEX 7C |
| 2 | Roll ID (Unit_ID) | 12 | 0123456789aa |  |
| — | Delimiter | 1 | ¦ | HEX 7C |
| — | End of MSG Char | 1 | ETX | HEX 03 |

# 5. Messages when Product at Labeling Station

## Operation

- PLC sends the product id and printer number to MES.

- MES prints the label via the local Windows driver.

- MES sends a message to PLCinforming that the label is printed or reporting an error.

- PLC sends a request for each required label.

Communication Flow:

- PLC -> MES: (1) "RE01" Roll at Labelling station

- MES -> LBL: (2) MES prints label if roll found

- MES -> PLC: (3) "ME01" Roll Label Printed

### "RE01" Roll is identified at Labeling station (MES ↔ PLC)

| No | Description | Length | Format | Comment |
| --- | --- | --- | --- | --- |
| — | Start of MSG Char | 1 | STX | HEX 02 |
| 1 | Message ID | 4 | RE01 |  |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| 2 | Roll ID (Unit_ID) | 12 | 0123456789af |  |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| 3 | Label printer | 1 | 0-1 | Printer number |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| — | End of MSG Char | 1 | ETX | HEX 03 |

### "ME01" Roll label printed (MES ↔ PLC)

| No | Description | Length | Format | Comment |
| --- | --- | --- | --- | --- |
| — | Start of MSG Char | 1 | STX | HEX 02 |
| 1 | Message ID | 4 | ME01 |  |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| 2 | Roll ID (Unit_ID) | 12 | 0123456789aa |  |
| — | Delimiter | 1 | ¦ | HEX 7C |
| 3 | Status | 1 | 0-1 | OK, FAULT |
| — | Delimiter | 1 | ¦ | HEX 7C (Pipe) |
| — | End of MSG Char | 1 | ETX | HEX 03 |